# ESP32 Signal Generator — Clean Reference of Implemented Changes (LVGL‑Excluded)

Scope: This document captures the restructuring and enhancements introduced, excluding all LVGL-related items and any LVGL wiring in main.cpp. It highlights what was added/modified, why it matters, and how to use the new pieces safely.

# 1. Overview & Scope

• Focused areas: code organization into a reusable library, serial CLI improvements, concurrency/safety architecture, and file inventory.

• Explicitly excluded: any LVGL UI layers, display drivers, UI callbacks, LVGL task pumps, and platformio.ini LVGL dependencies.

• Design goals: encapsulation of time-critical toggling, clearer APIs for configuration, safer interaction via queues, and easier maintenance.

# 2. Library Extraction — lib/ckp_gen

Codex extracted the crank/signal generator into a dedicated library to isolate time-critical code and present a clean public API.

2.1 Components

• CkpGenerator.h/.cpp — Core library exposing a concise API and hiding ISR details.

• Types provided:

– GapPosition: enumerates where the gap occurs within a period (e.g., start, middle, end).

– SignalConfig: holds user-facing parameters for the generated pattern (e.g., teeth count, gap size/position, RPM or period µs, logic level).

– IGenerator: interface/contract describing the generator behaviour.

– TimerCkpGenerator: concrete implementation using a hardware timer ISR for deterministic slot toggling.

2.2 Public API (summary)

// Construction & initialization
bool begin(int gpioPin);        // Configure output GPIO once; prepare timer/ISR
bool apply(const SignalConfig& cfg);
// Apply a new pattern atomically (period, teeth, gap slots/position, levels)

void start();                   // Arm the timer/ISR and start toggling
void stop();                    // Disarm the timer/ISR; safe to call repeatedly

// Read-only accessors (safe snapshots for non-ISR code)
bool readCkpLevel() const;      // Current logical output level
bool readGapWindow() const;     // True if we're inside the gap window
uint32_t getSlotPeriodUs() const; // Current slot period in microseconds

2.3 Rationale & Safety Notes

• Encapsulation: timing math, GPIO writes, and critical sections live in one place—reduces duplication and ODR issues.

• Determinism: the ISR toggles using precomputed slot timing; REG_WRITE is used for predictable latency.

• Concurrency: shared state (e.g., level, gapWindow, tick counters) is protected with portMUX-based critical sections to avoid races.

• API boundaries: configuration/state are applied via apply() and safe getters; clients do not poke ISR internals.

# 3. main.cpp (non‑LVGL) Changes

• Switched to using the library headers (CkpGenerator.h) instead of defining generator classes inline.

• Wrapped legacy in-file generator definitions with #if 0 to prevent redefinition and preserve the old implementation for reference.

• Kept the existing manager/queue control plane intact (see §4).

# 4. Serial CLI Enhancements

A friendlier, discoverable command-line interface on the serial port was introduced to simplify interaction without altering signal-generation logic.

4.1 New helper functions

• printHelp, printPatterns, printStatus, printPrompt — centralized output and a consistent prompt.

• pollSerialForDemo — parses commands and enqueues control messages.

4.2 Supported commands & aliases

• help / ? / h — show command overview.

• status — show current pattern, slot period, gap window, etc.

• patterns — list preset patterns by index.

• Bare-number RPM (e.g., "1500") — set RPM directly.

• Aliases (examples): "rpm=", "set rpm", "p", "pat" — quick access to common actions.

4.3 Interaction pattern (non-blocking)

• The CLI acts as a producer: it parses user input and enqueues a CtrlMsg into a FreeRTOS queue.

• The manager task acts as a consumer: it applies SignalConfig via the generator API, off the ISR path.

• Outcome: clean separation between UI/IO and timing-critical generation.

// Example control flow (conceptual)
void pollSerialForDemo() {
  // parse input...
  CtrlMsg m = {/*type/value or config*/};
  xQueueSend(gCtrlQ, &m, 0);      // produce
}

void managerTask(void*) {
  for (;;) {
    CtrlMsg m;
    if (xQueueReceive(gCtrlQ, &m, portMAX_DELAY)) {
      // apply configuration atomically on the generator
      // (start/stop/apply as requested)
    }
  }
}

# 5. Concurrency & Safety

• ISR discipline: ISR performs minimal, deterministic work (slot advance and GPIO toggle).

• Critical sections: portENTER_CRITICAL_ISR protects shared fields accessed by both ISR and tasks.

• Data handoff: producers (CLI) never touch ISR state; they enqueue immutable messages; the manager applies changes.

• Timing visibility: getters (readCkpLevel, readGapWindow, getSlotPeriodUs) expose snapshots without violating ISR constraints.

# 6. File Inventory (LVGL‑Excluded)

Added

• lib/ckp_gen/CkpGenerator.h — public API definitions (GapPosition, SignalConfig, IGenerator).

• lib/ckp_gen/CkpGenerator.cpp — ISR-backed implementation (TimerCkpGenerator).

Modified

• src/main.cpp — switched to library usage, added serial CLI helpers, and disabled old inline generator code via #if 0.

• platformio.ini — no non-LVGL-specific changes required in scope of this document.

# 7. Behavioural Summary (LVGL‑Excluded)

• Serial UX: users get a banner, help, patterns list, and a consistent prompt. Commands accept natural aliases and bare RPM entries.

• Signal behaviour: generation and timing remain functionally identical; only the interaction layer and code organization changed.

• Operational safety: deterministic ISR and queue-based control reduce coupling and prevent data races.

# 8. Quick API Reference

// Library types (names as provided)
enum class GapPosition { Start, Middle, End /*...*/ };

struct SignalConfig {
  // representative fields (actual fields per your header):
  uint16_t teeth;        // total slots/teeth per period
  uint16_t gapSlots;     // number of missing slots
  GapPosition gapPos;    // where the gap occurs
  uint32_t slotPeriodUs; // or derive from RPM
  bool highLevel;        // logical level for "tooth"
};

struct IGenerator {
  virtual bool begin(int gpioPin) = 0;
  virtual bool apply(const SignalConfig& cfg) = 0;
  virtual void start() = 0;
  virtual void stop() = 0;
  virtual bool readCkpLevel() const = 0;
  virtual bool readGapWindow() const = 0;
  virtual uint32_t getSlotPeriodUs() const = 0;
  virtual ~IGenerator() {}
};

// Concrete: TimerCkpGenerator (uses hardware timer + ISR)